Progress Report on

Development of Soy-formulate for organic ammonia production via hyperammonia-bacteria fermentation in a one-pot system

By

Ademola Hammed

Statements of goals/objectives

Our goal is to develop an alternative manufacturing method that could eliminate the CO2 footprint which can be employed by local farmers, thus, bringing ammonia close to where it is needed to reduce the emissions associated with both the production and distribution.

- Development of soy-based substrate for the production of green ammonia

Deliverables

Ammonia is very critical to the manufacturing of fertilizers. The industrial production of most of the world’s fertilizer using the Haber-Bosch process require huge amounts of energy to generate the high temperatures and pressures needed to combine nitrogen and hydrogen into ammonia. The high cost of transporting fertilizer from large manufacturing facilities to remote rural areas, such as sub-Saharan Africa is another herculean task, making it hard to be obtained in such areas. It is, therefore, necessary to develop a smaller-scale alternative that could be used locally to produce fertilizer for farmers or a small community of farmers.

The hydrogen gas used for the Haber-Bosch process is usually obtained from methane derived from natural gas or other fossil fuels. High temperatures (500 degrees Celsius) and pressures (200 atmospheres) are required to make the very unreactive nitrogen react with hydrogen to form ammonia. The process is not only very expensive but also emits a great deal of carbon dioxide, thus, making ammonia the largest contributor to greenhouse gas emissions, among all chemicals produced in large volumes.

Milestones and key performance indicators with

- Hydrolysis of soybeans

- Effect of different factors affecting soybean conversion to ammonia

Brief summaries progress, accomplishments and deliverables

Effect of substrate composition on microbial growth and ammonia production

The composition of substrate is among the major factors that affect fermentation process. Both protein and carbohydrate can provide carbon source for microbial proliferation. On one hand, bacteria generally prefer carbohydrates, primarily glucose for their growth. On the other hand, protein contain nitrogen that provide the amino base for ammonia production. To assess whether HAB can produce ammonia in cultures containing sugars (glucose or starch) only and sugars with a nitrogen source (casamino acid), it was observed that sugars alone could not support the growth of HAB; both starch and glucose give relatively poor growth of the bacteria (Figure 2). A similar result was seen in ammonia production when only sugars were used as the substrates. Nearly zero ammonia was produced in glucose only and starch only cultures (Figure 3). When the carbohydrate-containing cultures were supplemented with proteins, growth of the HAB surged (Figure 2), but not with concomitant high production of ammonia (Figure 4). This result showed that sugar inhibits ammonia production even when nitrogen-containing substrates are present. This is further evident as soybean flour which contains about 15% percent of sugars and more than 40% of protein among other components yielded low ammonia relative to peptide and casamino acid alone when each was used as substrates in different cultures (Figure 5).

Figure 2: Effect of different substrate on HAB growth profile. SYB: soybeans, CSA: Casamino, PEP: peptone, GLU: glucose, STH: starch: C+G: casamino and glucose and C+S: casamino and starch.

Figure 3: Ammonia production from carbohydrate substrate

Figure 4: Ammonia production from casamino + sugar substrate

Figure 5: Ammonia production from protein substrate

Protein isolation and enzymatic hydrolysis enhance ammonia synthesis

Having discover that sugar inhibit ammonia synthesis and peptide/amino acids support early ammonia synthesis, we conducted the effect of protein isolation and enzymatic hydrolysis on ammonia production during HAB fermentation. According to Figure 6, a reduced ammonia concentration was observed in cultures containing soybean flour. But when soybean protein isolate (SPI) was used as the substrates, a relatively high concentration of ammonia was obtained in the culture. The rate of ammonia production was much higher in hydrolyzed protein (HP) cultures, but both SPI and HP cultures performed better vis-à-vis ammonia production relative to soybean flour, reaching a concentration of 1200 mg/L (Figure 6). Soybean flour showed a delay in ammonia production. This is expected due to the presence of non-protein components such as sugars, oil, and flavones, amongst others. Some bacteria are known to digest large biological molecules into useful products without prior pretreatment or degradation of such large polymers. Caldicellulosiruptor species is a classic