Bioconversion and bioaugmentation of soybean meal for sustainable biofertilizer production

Ademola Hammed, Niloy Chandra Sarker, Owumbua Monono

Agricultural and Biosystems Engineering, North Dakota State University

Research Overview and Objectives: background information and research gaps

The expanding global population has increased the demand for sustainable biofertilizer sources, driving the exploration of alternative methods to traditional agricultural practices. Microbial fermentation has emerged as a promising alternative due to its potential for high yield and low environmental impact. However, conventional fertilizer production and using carbon (glucose) and nitrogen (ammonia, urea) sources in fermentation raises significant environmental and economic concerns, such as high-energy consumption and greenhouse gas emissions. Our study utilizes lignocellulosic biomass, a versatile and sustainable agricultural residue, as the primary feedstock (carbon source) for fermentation to increase protein value. The microbial protein (MP) is expected to be used for biofertilizer production using previous methods that have been developed by our research group. Additionally, we use soybean root-noodle microbes - K. oxytoca M5A1 - that can fix atmospheric nitrogen using inherent Nitrogenase. Nitrogenase reduces atmospheric nitrogen (N2) by breaking the strong triple bond between nitrogen atoms, resulting in the formation of ammonia (NH3) [1]. K. oxytoca M5a1 is of interest because it exhibits potential in producing beneficial chemicals and biofuels from a wide range of sugars, and it also plays a role in nitrogen fixation [2]. It is acknowledged as a nonpathogenic species of Klebsiella [3]. We envisage that the use of switchgrass as substrates reduces the cost of raw materials, while the nitrogen-fixing capability of bacteria eliminates the need nitrogen sources, thereby lowering the fermentation costs during MP synthesis. This approach would potentially not only makes MP production more economically viable but also aligns with global efforts to reduce the environmental footprint of food/feed production.

Materials and Methods

Media

- Luria-Bertani agar (LB agar): 5 g tryptone, 7.5 g agar, 2.5 g yeast extract, 5 g sodium chloride (NaCl) per 500 ml distilled water.

- Minimal fermentation medium: 0.1 g CaCl2−2H2O, 25 g Na2HPO4·7H20, 0.25 g MgSO4·7H2O, 3 g KH2PO4, 0.25 mg Na2MoO4·2H2O, 1 g NaCl, 2.9 mg FeCl2·4H2O per liter of distilled water.

- Super Optimal Broth (SOB) medium: 0.5 g NaCl, 2.4 g MgSO4, 5 g yeast extract, 0.186 g KCl, 20 g Peptone per liter of distilled water. Sucrose solution was prepared for control experiment.

Culturing of Klebsiella oxytoca M5A1

- A fresh culture of bio-engineered K. oxytoca M5A1 was sub-cultured on LB agar plate.

- Using a sterile wire loop, a distinct colony of K. oxytoca M5A1 was carefully streaked on the LB agar plate under aseptic condition and the plates were incubated for 24 h at 30°C.

- After incubation the overnight culture was prepared by aseptically introducing a swab of K. oxytoca M5A1 into 10ml SOB broth and incubated for 24 h (30°C, 200rpm).

Biomass Pretreatment and Enzymatic Hydrolysis:

- The lignocellulosic biomass (Switchgrass) was pretreated using alkaline aqueous ammonia (1:6 solid-liquid ratio of 10% (w/v)). The mixture was incubated at 60°C for 24 hours. After incubation, the pretreated samples was washed and air-dried on a workbench for several hours.

- Following pretreatment, the alkaline-pretreated switchgrass was hydrolysis using Cellic HTec cellulase enzyme (Novozymes, Denmark). The enzymatic hydrolysis occurred for 72 hours at 50°C with agitation at 130 rpm. Upon completion, the resulting switchgrass hydrolysate was filtered to remove residues and autoclaved for 20 minutes to maintain sterility.

Analytical Methods: Glucose Utilization and Organic Acids Production

- The glucose content in the hydrolyzed biomass, glucose utilization and organic acids produced during fermentation were analyzed and quantified with HPLC system using LabSolutions Software (version 5.57) for Shimadzu LC-2050C HPLC.

- Different concentrations of glucose and organic acids were equally run to generate standard curves, respectively. The samples (glucose and organic acids) were quantified using external standards and results were reported in g/L.

Fermentation Conditions and Growth Profiling

- The fermentation media consisting switchgrass hydrolysate (pH 6.5-7.5) was inoculated with 5% of overnight culture and incubated for 48 h (30°C, 200rpm).

- The samples were collected at 0, 8, 16, 24, 32, 40, and 48 h, and the optical density of each collected samples were taken (OD 600nm).

- Microbial growth was determined with the use of microplate Spectrophotometer (Infinite M Nano by Tecan).

Protein Quantification

- The amount of protein produced by K. oxytoca M5A1 was quantified at each time interval using Bradford assay protein quantification protocol (standard curve was established using the Bovine Serum Albumin assay).

- A control experiment was performed using sucrose as a carbon source. This comparison aims to assess the efficiency of protein yield from K. oxytoca M5A1 when grown on switchgrass hydrolysate compared to traditional methods of microbial protein production.

- Bradford reagent was added in ratio 1:5 v/v. The mixture was mixed and incubated at room temperature for 20min and the absorbance was taken at wavelength of 595nm using Tecan infinite M Nano microplate reader.

Acetylene Reduction Assay

- Nitrogen fixing ability of K.oxytoca M5A1 was determined by acetylene reduction assay (ARA) as described by Wen et al (2021) with some modifications.

- K. oxytoca M5A1 was cultured in two substrates; fermentation with switchgrass hydrolysate (FwSH), and fermentation with sucrose (FwS) as control.

- The analysis was done by HP 6890 gas chromatography system (Agilent Technologies, Inc, Santa Clara, California, U.S.) with a flame ionization detector.

- The quantity of ethylene produced was determined by using the ethylene peak area for ARA calculations, and then converted to molar concentration of ethylene.

Statistical analysis

- The findings were presented as the mean ± standard deviation of three replicates, and statistical analyses were conducted using the Minitab statistical software 21.

- Data were subjected to Analysis of variance (ANOVA), followed by Tukey’s multiple comparison test to identify significant differences at a significance level of p < 0.05.

- Krus-kal-Wallis test, followed by Mann-Whitney U tests with Bonferroni correction were used where normality assumptions were not met.

Research Results/Outcomes

3.1. Growth Profile K. oxytoca M5A1 in switchgrass hydrolysate

Fermentation process was carried out at 30 °C for 48 h, under the pH of 6.5 and 200 rpm. The medium contained 10 g/L equivalent of glucose from the hydrolysate, and we monitored the growth profile of the bacterial strain with 8 h intervals for 48 h.

Findings

- Initial rapid growth phase was observed from 0 to 8 hours, with an increase from 0.18 to 0.60 (OD600nm).

- Beyond 8 h, the growth rate appears to slow down, with a consistent and steady increase in the average values.

- The highest average value was observed at 48 h with absorbance of 0.78.

- Absorbance values at time points 0, 8, 16, and 24 were significantly lower compared to time points 32, 40, and 48 (p < 0.05). (Figure 1)

Figure 1. Growth profile of nitrogen-fixing K. oxytoca M5A1 in Switchgrass hydrolysate. The data are presented with mean values ± SD of n=3 experiments.

3.2. Glucose Utilization of K. oxytoca M5A during Fermentation

To ascertain whether the previously documented rapid growth correlated with the glucose utilization rate of our strain, we monitored the glucose consumption rate of K. oxytoca M5A1 utilizing a HPLC system. The samples were taken at time intervals (every 8 h) and the glucose concentration was estimated using external standards (Figure 2).

Outcome

- The results obtained indicated a significant decrease in glucose concentration from 12.83 g/L to 3.80 g/L within the first 8 h.

- Complete depletion of glucose was observed by 16 hours.

- Significant differences in glucose concentrations among time points were detected (p < 0.05).

Figure 2. Utilization rate of glucose during fermentation. The data are presented with mean values ± SD of n=3 experiments.

MP Content of K. oxytoca M5A1

We use sucrose as a control experiment for a direct comparison with switchgrass hydrolysate. The production of microbial protein was carefully monitored at time interval using Bradford protein quantification assay at wavelength of 595nm. Microbial protein concentration of K. oxytoca M5A1 in switchgrass hydrolysate (FwSH) and sucrose (FwS) (Figure 3).

Outcome

- The initial phase (0 – 8 hours) showed an upward trend in protein concentration (FwSH: 53.41 to 284.73 µg/ml; FwS: 18.86 to 220.05 µg/ml).

- Further increase in protein concentration was noted at the highest level at 40 hours (FwSH: 683.46 µg/ml; FwS: 625.65 µg/ml).

- At 48 h, it was also established that there is a drop in protein concentrations (FwSH: 376.98 μg/mL; FwS: 301.83 μg/mL).

- On the other hand, there were higher protein levels in FwSH than in FwS (mean difference = 46.31 µg/mL, p <0.05).

Figure 3. Microbial protein concentration of K. oxytoca M5A1 in switchgrass hydrolysate (FwSH) and sucrose (FwS).

Nitrogen-fixing capability of K. oxytoca M5A1

Here, we conducted ARA to measure the nitrogen-fixing activity of K. oxytoca M5A1 cultured in switchgrass hydrolysate (FwSH) and sucrose (FwS) to investigate how different carbon sources affect the efficiency of nitrogen fixation (Fig 5). The sampling was done at 6 h interval for over 30 h using HP 6890 GC system (2min method). No Ethylene was present in both treatments at 6 h (Figure 4).

Outcome

- Ethylene production commenced at 12 hours in both FwSH and FwS cultures.

- FwS showed higher acetylene reduction (4.62-109.43 µmoles) compared to FwSH (3.65-69.82 µmoles).

- No significant differences in ethylene production between the substrates were found (p > 0.05)

Figure 4: Nitrogen-fixing capability of K. oxytoca M5A1 cultured in FwSH and FwS. Error bars represent the standard deviation (n=2).

Organic Acids Profile during Fermentation to Protein

An understanding of organic acid profile during fermentation will reveal the active metabolic pathway and activities. Since the metabolic dynamics of organic acid during nitrogen fixation of K. oxytoca M5A1 is unclear, we studied the production of organic acid over the course of fermentation. We observed the presence of lactic acid (LA), succinic acid (SA), acetic acid (AA), and propionic acid (PA) during the fermentation process (Figure 5).

Outcome

- Initial concentrations of LA and PA were 0.99 g/L and 2.06 g/L, respectively.

- PA showed the highest and continuous increase until 24 hours, LA increased slightly until 24 hours, SA increased steadily until 24 hours then declined, and AA synthesis began after 8 hours and continued until 24 hours.

- LA, SA, and AA were undetectable at 32 hours, but SA and AA reappeared later.

- Significant differences in organic acid concentrations over time were found (p < 0.05).

Figure 5. Organic acids profile during fermentation. The data are presented with mean values of organic acids concentration ± SD of n=3 experiments.

4.	Discussion

Evidence from our growth profile study indicated that switch grass hydrolysate could be efficiently exploited by K. oxytoca M5A1 as a carbon source for fermentation. The bacteria grew rapidly in the first eight hours of fermentation as shown by an increase in absorbance at OD600nm. The fast growth of this stage can be explained by the presence of nutrients within the hydrolysate. However, there was a decrease in growth rate during the following hours, indicating nutrient depletion as a limiting factor. This observation is crucial for optimizing fermentation conditions, which indicate the need for supplementing nutrients to sustain prolonged microbial activity and productivity in protein production. Efficient metabolic capabilities of M5A1 were revealed when glucose utilization was studied on K. oxytoca M5A1. The rapid consumption of glucose within 16h coupled with biomass increment shows how good it is at quick conversion of sugar to biomass and other metabolic products. Such efficiency helps maximize protein yield from biomass feedstock thus minimizing substrate wastage and improving overall process economics. It is worth noting that understanding such kinetics can also help to keep fermentation processes under control so that they can have optimal productivity and resource efficiency.

Klebsiella oxytoca M5A1’s protein synthesis in switchgrass hydrolysate varied significantly over time due to both carbon availability and metabolic activity changes . This shows that proteins are synthesized more during initial growth stages because cells get stronger with available nutrition obtained from hydrolysis sources. The observed differences between switchgrass hydrolysate and sucrose (as a control) highlight the unique nutritional benefits of hydrolysate for microbial protein production. These results are imperative for biotechnological approaches aimed at enhancing protein yield from agro-waste materials which can potentially contribute to sustainable animal feed or biopharmaceuticals based-production. The nitrogen-fixing capability study using Acetylene Reduction Assay (ARA) confirmed the capability of K. oxytoca M5A1 to fix nitrogen in switchgrass hydrolysate and sucrose. The rate at which ethylene is produced between the two carbon sources differs though, but statistical analysis showed no significant difference that implies it has consistent nitrogen-fixing capabilities for various nutritional conditions. This results are very important when it comes to protein production as they help reduce the amount of external nitrogen being used, thereby making it more sustainable and environmentally friendly.

Metabolic pathways and substrate utilization patterns were revealed by analyzing organic acids produced during fermentation for K. oxytoca M5A1. A variety of lactic acid, succinic acid, acetic acid and propionic acid profiles over time indicate strong adaptability of this strain under different fermentation conditions. Understanding organic acid dynamics can assist in optimizing fermentation parameters for achieving maximum yield of desired products with minimum by-products too. Such information is necessary for designing specific biotechnological applications like bioproducts manufacturing or chemical synthesis through fermentation.

5.	Conclusion/Benefits to the North Dakota Soybean Farmers and the Industry

The nitrogen fixing bacteria K. oxytoca M5A1 is highly efficient in utilizing switchgrass hydrolysate, a lignocellulosic biomass, for microbial protein production thus providing an alternative to the traditional glucose and nitrogen sources which are not sustainable. The organism demonstrates complex metabolic pathways and adaptation to nutrient availability. These results have great significance for industry as they promote ecological integrity, thereby aligning with global environmental initiatives that encourage sustainability while ensuring economic soundness in production processes. To North Dakota soybean farmers, MP production integration brings new avenues of money from soybean byproducts. Microbial fermentation can be supported by products realized after processing soybeans, like hulls alongside other plant remains. Soybean residues thus become more valuable and different revenues streams are created for farmers. Consequently, this will reduce dependence on soybean-based feeds and create a diversified agricultural economy because the produced MP is also usable as high-quality livestock feed. This not only develops scientific knowledge but also offers practical applications thereby contributing to improved productivity and sustainability for the farmers of soybean and the whole industry as well.

References

1. 	Burén, S.; Rubio, L.M. State of the Art in Eukaryotic Nitrogenase Engineering. FEMS Microbiol. Lett. 2018, 365, doi:10.1093/femsle/fnx274.

2. 	Bao, G.; Zhang, Y.; Du, C.; Chen, Z.; Li, Y.; Cao, Z.; Ma, Y. Genome Sequence of Klebsiella Oxytoca M5al, a Promising Strain for Nitrogen Fixation and Chemical Production. Genome Announc. 2013, 1, e00074-12, doi:10.1128/genomeA.00074-12.

3. 	In, S.; Khunnonkwao, P.; Wong, N.; Phosiran, C.; Jantama, S.S.; Jantama, K. Combining Metabolic Engineering and Evolutionary Adaptation in Klebsiella Oxytoca KMS004 to Significantly Improve Optically Pure D-(−)-Lactic Acid Yield and Specific Productivity in Low Nutrient Medium. Appl. Microbiol. Biotechnol. 2020, 104, 9565–9579, doi:10.1007/s00253-020-10933-0.