28% protein Soy Based Cattle Cube Processing Efficiency and Finished Product Quality

Shane Mueller
Northern Crops Institute | North Dakota State University
Fargo, ND

Abstract

Four different formulas to produce a 28% crude protein range cattle cube were tested. The results are written and discussed in two sets as two of the formulas would not run through the original pellet die chosen. Formula one (dried distillers’ grains with solubles) and formula two (soybean meal and soybean hulls) were ran on the same equipment. Formula three (soybean meal, soybean hulls, and wheat middlings) and formula four (soybean meal and wheat middlings) were ran on a different pellet mill die. All four formulations were set by using guaranteed analysis specifications to get the final crude protein content to 28%. Formula two showed a better production rate in tons per hour than formula one did. When looking at formula three and four the production times and rate were identical but formula four visually had more fines produced than formula three. In terms of energy usage formula two showed an advantage over formula one. In the second set of formulas, four had an advantage over three. In terms of pellet quality formula two again outdid formula one. Formula four showed slightly better quality than formula three. Final product quality, energy usage, and product price may all be key driving factors moving forward for producers to pick feedstuffs.

Introduction

With three new soybean processing plants being built in North Dakota there is going to be a large volume of local soybean meal. New plants and plant expansions are also being constructed throughout the United States Corn Belt. With a limited number of monogastric animals being raised in North Dakota ruminants may be a good fit for using this soybean meal supply. Soybean meal not consumed locally will be exported though the Pacific Northwest. While at some point more monogastric confinement complexes may be built in this region right now with inflation, building costs, outside disease presence, and market prices for finished animals it does not make sense for those buildings to go up. With almost 900,000 head of beef cows in North Dakota and the predominant majority being in Western North Dakota, which is subject to drought, range cubes are a common feed choice to supplement protein to range cattle.

When selling range cubes to cattle producers, two of the most important characteristics are price and quality. Another area coming to focus on sustainability is the energy used to produce feed. For this project, the energy of the pellet mill motor was monitored to see differences in electrical energy usage. When it came time to determine formulas to evaluate there were four different formulations chosen. The targeted comparison was between a complete dried distillers’ grains with solubles cube, which is currently commercially made, and a cube made solely of soybean meal and soy hulls. I could find little on any commercial offerings for a soybean meal and soy hull-based cube. In the southern United States, many mills offered a combination of soybean meal and cotton seed hulls but not soybean meal and soybean hulls. The two formulas that contained wheat middlings were brought in to evaluate energy differences with the addition of the wheat middlings and pellet durability. Wheat middlings commonly have some starch present and will normally make a slightly better pellet.

Materials and Methods

Four formulas were used for this project. The base formula was pure dried distillers’ grains with solubles. Formula two was 52.5% soybean meal and 47.5% soyhulls, formula three was 47% soybean meal, 26.5% soyhulls, and 26.5% wheat middlings, and lastly four was 40% soybean meal and 60% wheat middlings. The original plan was to run all four formulas on our 5/8” step die. Formulas three and four were not able to run on the 5/8” die. Formulas three and four were made on a ½” die with different specifications which can be seen in figure 1.

Two tons of each formula was produced on our California Pellet Mill brand pellet mill. The mill at Northern Crops Institute Feed Production Center is a 12” ring die Hy-Flow model. Parameters evaluated during the production stage included total run time, total kWh used, feed temperature before and after conditioning. In the lab moisture levels prior to conditioning and after conditioning were evaluated as well as pellet durability index with the tumble box method and density of the starting mash and ending pellet products.

The production rate was done by recording the starting and ending times of each production run while operating the pellet mill. Two tons of feed was then divided by that time to give us the production rate in tons/hour. Kilowatt-hours were recorded with an AMEC PEL 103 data logger. The logger is set to record 480 volt 3-phase power utilized at Northern Crops Institute Feed Production Center. The meter recording was started prior to the production of pellets and the start and end times of the production run were used to analysis the window where pellets were being produced. This meter records through Bluetooth to a computer and captures all the energy data needed including the total kilowatt-hours used. From there, the total kilowatt-hours used was divided by two tons to get kilowatt-hours per ton. The cold mass temperature and the conditioned temperature are taken from digital gauges on the pellet mill controls. The estimated moisture addition is done by using the calculation of every 16.1° C change in temperature from the steam adds 1% moisture.

For the lab testing that was done in the Northern Crops Institute feed lab. An OHAUS MB 35 moisture analyzer was used to check for moisture levels in the mash. This equipment is like what a commercial feed mill may have available. It was set to run the auto program at 100° C. The cold moisture sample was taken from the bin prior to starting the pelleting process. The conditioned sample was taken from the transition between the conditioner and pellet mill. The sample was analyzed immediately after it was taken. The cooled moisture sample was done by taking cooled pellets as they were discharged into the truck and crushing them down to a mash for analysis. The pellet durability index, due to the size of the products produced, was done with the tumble box method. 500 grams of cleaned pellets all over 1” in length were put into the tumble box for ten minutes. At that time, the remaining good quality product was screened to remove fines and unders and re-weighed to determine pellet quality. Lastly the density checks were done two separate ways. The mash was done with a 1 quart measuring cup from OHAUS and a grain grading scale 8800SS from Seedburo. For the finished product density, a 5-gal bucket was used with a determined volume. The bucket was filled level to the top as if product were dropped into a bin. From there the pre-determined volume and net weight in the bucket was used to determine density.

Figure 1: Die specifications used.

| Parameter | (1) DDGS 5/8” | (2) SBM/SH 5/8” | (3) SBM/SH/WM ½” | (4) SBM/WM ½” |
| --- | --- | --- | --- | --- |
| Time for 2 short ton | 3:05 | 2:36 | 1:09 | 1:09 |
| Tons/HR | .65 | .82 | 1.73 | 1.73 |
| kWh Used | 61.184 | 56.27 | 21.419 | 18.361 |
| kWh/Ton | 30.59 | 28.135 | 10.71 | 9.18 |
| Cold Mash Temp | 2.3 C | 2.1 C | -5.2 C | -4.7 C |
| Conditioned Temp | 43 C | 62 C | 60.3 C | 61 C |
| Calculated Moisture Addition | 2.56% | 3.73% | 4.07% | 4.08% |
| OHAUS Cold Moisture | 11.42% | 11.2% Dry 14.08% with water added | 11.78% | 11.86% |
| OHAUS Conditioned Moisture | 13.34% | 15.92% | 15.14% | 14.32% |
| OHAUS Cooled Moisture | 11.08% | 13.84% | N/A | N/A |
| Tumble Box PDI | 79.90% and 79.85% | 86.89% and 82.84% | 84.7% and 84.7% | 92.1% and 93.7% |
| Diet Cost Per Ton | $210.00 | $262.35 | $318.29 | $244.60 |

Table 1: Observed and calculated results from cube production on the Hy-Flow Pellet Mill

Results and Discussion

The results of formula one and formula two will be discussed first. These two formulas were produced with the 5/8” step die. Formula two, consisting of soybean meal and soy hulls produced two tons of feed in just over two and a half hours. This gave a production rate of 0.82 tons per hour. Formula one, consisting of pure DDGS (Dried Distillers Grains with Solubles), produced two tons of feed in just over three hours at .65 tons per hour. Formula two outperformed formula one in energy consumption consuming 28.135 kWh/ton compared to 30.59 kWh/ton. When it comes to pellet quality formula two showed an advantage in pellet durability index at 86.89% and 82.84% compared with 79.90 and 79.85% for formula one. Both formulas showed good drying characteristics when analyzed for moisture content. Formula one has a price advantage over formula two of $52.35 per ton.

For formula three and formula four comparable results were obtained. Both formulas ran two tons of feed in one hour and nine minutes at a pace of 1.73 tons per hour. Formula four had a slight advantage in kWh/ton at 18.361 kWh/ton compared to 21.419 kWh/ton for formula three. Formula three showed a higher moisture level at an almost identical conditioning temperature leading to the assumption it took better heat transfer from the steam conditioning. That would lead one to expect a better pellet durability, but formula four showed an advantage over formula three with a 7-8% higher durability index.

Producing a quality pellet can be broken down into a list of influential factors. Proper pellet conditioning, quality steam production and proper retention time are the largest factors determining pellet quality. This is followed by formulation, particle size, die specifications, and cooling or drying of pellets. All the products used in this project have a reputation for not being easily pelleted with the exception, to an extent, of wheat middlings. Soybean meal, DDGS, and soyhulls individually are not exceptionally easy to pellet. During the ethanol boom work was done at several universities to understand how DDGS itself and mixed formulations affected feed manufacturing. The short answer to that question was that as DDGS levels are increased pellet production is slowed. While that is not fully understood it is attributed to the zein protein complex along with other contributing factors. To create exceptional quality pellets, you need a balance of starch and protein to bind together. Starch during the conditioning phase becomes a more glue-like substance and binds well with protein. In these formulations there is little starch present. Fiber does not pellet will, as it has a natural resistance to being compressed into the pellet die and protein by itself does not bind well for pelleting. High protein, low starch feeds can be tricky to produce into pellets.

When analyzing the information found in this project, a key takeaway is that under the same pellet mill die specifications a soybean meal and soyhull-based cube outperformed the DDGS cube. Market conditions right now are not optimal for the production and expansion of the swine or poultry industry. Cattle are already present in large numbers throughout North Dakota, often being raised in drought prone areas. During this next year soybean crush facilities will start operating in the state, reducing transportation cost of the soy ingredients. Depending on where commodity prices land, a soy-based cube can be applied in the local region. This project has shown that a quality product can be produced. The second hurdle to overcome is price competitiveness in the open market. Most feed formulations are done on a least cost analysis. While it did take less energy resources to produce the soy-based cube in formula two, that will not be enough to offset the raw ingredient difference yet. As the industry expands and the market corrects itself for available product a higher use of soy in ruminant protein supplements could prove to be an excellent use of the resource.

The last piece of information to help provide value to the soy producers of this region will be the creation of an industry guidance flyer for feed mills that want to start producing a soy-based protein supplement option. There will be cannulated cattle data that follows this feed production write up from the animal science faculty here at NDSU. Upon NCI receiving that information it will be added to the feed production guidance flyer.