Executive Summary North Dakota Soy Council

Project: 28% protein Soy Based Cattle Cube Processing Efficiency and Finished Product Quality

Principal Investigator: Shane Mueller
Co-Investigator: Zachary Carlson

Executive Summary

This project, in collaboration with Zachary Carlson from NDSU (North Dakota State University) Animal Science, was to evaluate the effectiveness of producing a soy-based range cattle cube to compete in the higher protein cube market. The specific objectives in the feed mill were to understand how to process the soy-based cube, energy demands, and determine product quality. The end goal is to provide guidance to the industry on how to produce a soy-based range cube including equipment set up and handling.

Four formulas were used for this project. The base formula was pure dried distillers’ grains with solubles. Formula two was 52.5% soybean meal and 47.5% soyhulls, formula three was 47% soybean meal, 26.5% soyhulls, and 26.5% wheat middlings, and lastly four was 40% soybean meal and 60% wheat middlings. The original plan was to run all four formulas on our 5/8” step die. Formulas three and four were not able to run on the 5/8” die. Formulas three and four were made on a ½” die with different specifications. Due to using two different dies, we can compare formulas one and two and then separately three and four.  Two tons of each formula was run on our California Pellet Mill brand pellet mill. Start and end times were recorded, an AMEC data logger was used to track power use, and samples were taken throughout the runs to get the rest of our data. Price per ton was calculated off current market prices at the time of the project for each ingredient used.

The results of this project show promise that a soy-based cube can be a viable option. When comparing formula one with formula two production rate, pellet durability index, and energy use all showed favor to formula two. Neither formula ran as fast as anticipated. This could be attributed to pellet mill only being 50 horsepower and both DDGS (Dried Distillers Grains with Solubles) and soybean meal/soyhulls have a strong resistance to pelleting. These two were expected to run slower but still close to two tons per hour. Results between formulas three and four are similar with formula four having a slight advantage over formula three in kwh/ton and pellet durability index. Our most important result is that formula 2 is possible to produce and from an energy and PDI standpoint it can compete with the quality of a DDGS. The variable out of our control in the feed mill is ingredient prices.

| Parameter | (1) DDGS 5/8” | (2) SBM/SH 5/8” | (3) SBM/SH/WM ½” | (4) SBM/WM ½” |
| --- | --- | --- | --- | --- |
| Time for 2 short ton | 3:05 | 2:36 | 1:09 | 1:09 |
| Tons/HR | .65 | .82 | 1.73 | 1.73 |
| kWh Used | 61.184 | 56.27 | 21.419 | 18.361 |
| kWh/Ton | 30.59 | 28.135 | 10.71 | 9.18 |
| Cold Mash Temp | 2.3 C | 2.1 C | -5.2 C | -4.7 C |
| Conditioned Temp | 43 C | 62 C | 60.3 C | 61 C |
| Calculated Moisture Addition | 2.56% | 3.73% | 4.07% | 4.08% |
| OHAUS Cold Moisture | 11.42% | 11.2% Dry 14.08% with water added | 11.78% | 11.86% |
| OHAUS Conditioned Moisture | 13.34% | 15.92% | 15.14% | 14.32% |
| OHAUS Cooled Moisture | 11.08% | 13.84% | N/A | N/A |
| Tumble Box PDI | 79.90% and 79.85% | 86.89% and 82.84% | 84.7% and 84.7% | 92.1% and 93.7% |
| Diet Cost Per Ton | $210.00 | $262.35 | $318.29 | $244.60 |

Results from the processing in the feed mill for all four trials

Die specifications

1 ½”

2 ½ ”

1”

5/8”

2 ½”

½”

Formula 1 & 2 
4:1 L/D ratio
141.37in2 surface area
28.52in2 open area

Formula 3 & 4 
5:1 L/D ratio 
141.37in2 surface area
82.47in2 open area